BEDMAP database format for ice thickness and supporting data - Version 2

David Vaughan

Matthew Lythe

July 31, 2026

The following is a detailed description of the data format proposed for BEDMAP datasets.  The model was derived after discussions with Ian Allison (Antarctic CRC) and borrowing from an existing system implemented by APR Cooper in the BAS/SPRI/TUD map 'The Bedrock Surface of Antarctica'.  Minor changes have been made to the data format to enable compatibility with the Antarctic Data Directory System (ADDS).   All source BEDMAP data will be imported into an Oracle database on a Unix system residing at the British Antarctic Survey.  The Oracle database of ice thicknesses and surface elevations will in turn be exported into the Geographic Information System (GIS) Arc/Info.  These data together with the best available bathymetric data out to the continental slope and surface DEM will be combined into a continuous spatial data set data to be used to develop the gridded topography of bedrock.

The data contributed to BEDMAP are divided into missions to allow broad groupings of similar data.  The missions will generally consist of data from a group of sorties/flights for which acquisition parameters remained broadly similar. For each mission there will be main components or tables in the system

- Mission Data Table - the principal data table in BEDMAP.   A single unique row or record will be used to describe each point based measurement of ice thickness.  Fields include mission_id, sortie_id (flight number), latitude, longitude, ice thickness, surface elevation and internal references used to map back back to the original data set.

- Mission Summary Table - the principal metadata table containing information from each mission.  Each mission will have a unique record describing methods in data collection, navigation, data logging, processing, source contacts and availability.

- Trackline Sortie Data Table - specific to trackline data, one per sortie including a description of information unique to the sortie and a generalised version of the track which will enable rapid geographic searches of the database.  This table will be generated automatically from the Mission Data Table.

As specified in the BEDMAP report only final interpreted versions of the data should be submitted to the BEDMAP.  Data will be accepted in ASCII format or proprietary formats.  Each data file should contain measurements in column format.

Mission Data Table

All fields in this table are mandatory with a null value of -9999.0 being applied where data is unavailable.

| Meta data Field | Type | Field Width (Characters) | Description |
| --- | --- | --- | --- |
| RECORD_ID | NR | 4 | Record number referring to point based measurement of ice thickness. Primary key (unique) to data record |
| SORTIE_ID | NR | 5 | Applicable only to trackline data, sortie reference id assigned to sortie within the overall airborne mission (Value of -9999 applied to point based data) |
| MISSION_ID | NR | 20 | Mission reference ID number assigned by BEDMAP project, principal link to metadata table 'mission_summary' |
| LATITUDE | NR | 15 | Latitude of measurement, decimal degrees, +ve North |
| LONGITUDE | NR | 15 | Longitude of measurement, decimal degrees, +ve East |
| ICE_THICKNESS | NR | 4.1 | Measured ice thickness / m |
| SURFACE_ELEV | NR | 4.1 | Measured surface elevation / m - If surface elevation was measured in conjunction with ice thickness it will be included here |
| BEDROCK_ELEV | NR | 4.1 | Elevation of bedrock / m, determined from ice thickness and surface elevation measurements |
| INTERNAL_REF1 | CHAR | 20 | Reference code used by data providers e.g. Time, Distance, Cane |
| INTERNAL_REF2 | CHAR | 20 | Additional reference code used by data providers e.g. Time, Distance, Cane. |

Mission Summary Table

| Meta data Field | Type | Field Width (characters) | Description |
| --- | --- | --- | --- |
| PK | NR | 4 | Primary key (unique) to reference mission |
| MISSION_ ID | CHAR | 20 | Mission reference ID assigned by BEDMAP project |
| DESCRIPTION | CHAR | 255 | Description of the mission |
| REGION | CHAR | 100 | Description of the geographical extent of the mission (TransAntarctic Mts, Ross Ice Shelf, Marie Byrd Land) |
| START_DATE | DATE |  | Start date of the mission |
| STOP_DATE | DATE |  | Stop date of the mission |
| DATATYPE | CHAR | 30 | Keyword (Airborne RES / Ground-based RES / Seismic / Gravity / Borehole) |
| PLATFORM | CHAR | 150 | Description of platform from which measurements were made (eg. Twin Otter Aircraft) |
| THICMETH | CHAR | 200 | Method used to obtain ice thickness (eg. Seismic, RES, Gravity, Borehole) |
| THICPROC | CHAR | 400 | Description of algorithm used to calculate ice thickness (Including numeric constants) |
| THICLOG | CHAR | 500 | Description of data logging methods (Digital, Analogue, 35 mm file etc.) |
| THICPREC | NR | 4 | Estimated accuracy of ice thickness measurement / m |
| NAVMETH | CHAR | 100 | Keyword ( local fix / DR / Astro / Inertial / Doppler / GPS/ < other > ) |
| NAVDESC | CHAR | 300 | Description of navigation methods. |
| NAVPREC | NR | 4 | Estimated accuracy of navigation / m |
| TERRMETH | CHAR | 100 | Aircraft height about ground / m (Const. pressure, const. terrain clearance, etc.) |
| TERRLOG | CHAR | 300 | Description of surface elevation logging methods |
| TERRPREC | NR | 4 | Estimated accuracy of surface elevation method / m |
| SURFMETH | CHAR | 300 | Description of instrumentation, algorithms, etc. Orthometric or ellipsoidal |
| SURFLOG | CHAR | 300 | Description of logging methods for surface |
| SURFPREC | NR | 4 | Estimated accuracy of surface elevation measurements / m |

| DATUM | CHAR | 20 | Ellipsoid to which data are fixed |
| --- | --- | --- | --- |
| REFMAP | CHAR | 40 | Map sheet used in fixing etc. |
| DATAVOL | CHAR | 40 | Volume of data (no. of recordings) |
| SOURCELOC | CHAR | 300 | Institute and person holding the original data or best contact. |
| SOURCE_INST | CHAR | 100 | Agency(s) responsible for funding project |
| SOURCE_COUNTRY | CHAR | 50 | Country of principal source institution |
| SOURCEPI | CHAR | 300 | Information about the Principal investigator who collected these data |
| SOURCEREF | CHAR | 500 | References to publications that have discussed these data |
| AVAILABILITY | CHAR | 300 | Current availability of data and restrictions placed upon them |
| COMMENTS | CHAR | 500 | Other pertinent comments concerning data |
| LAST_UPDATED | DATE |  | Date data last updated in database |
| LAST_UPDATED_BY | CHAR | 40 | Information about the person who last updated the mission record(s) in database |

CHAR - free text, KW - Keyword, NR - Number, DATE - Date (dd-mm-yyyy)

In addition, to these fields it is desirable that some system parameters are recorded for each measuring system.

Trackline Sortie Data Table

This table contains the date and data volume as well as the generalised track for each sortie. It is intended to generate these simplified tracks using a Douglas-Peuker filter applied to the original data.  It will provide a useful simplified version of the data that will allow quick low resolution plotting of the flight tracks.  The table also provides a link to both the data table containing the ice thickness measurements and also the metadata pertaining to each mission.

| Meta data Field | Type | Description |
| --- | --- | --- |
| SORTIE_ID | NR | ID assigned to the sortie of |
| MISSION_ID | NR | Mission reference ID number assigned by BEDMAP project, link to metadata |
| DATE | DATE | Date of the sortie |
| DATAVOL | CHAR | Volume of data (no. of recordings) |
| LATITUDE_1 | NR | Beginning of Trackline latitude of measurement, decimal degrees, +ve North |
| LONGITUDE_1 | NR | Beginning of trackline longitude of measurement, decimal degrees, +ve East |
| LATITUDE_2 | NR | Second trackline latitude of measurement for generalised description of track |
| LONGITUDE_2 | NR | Second trackline longitude of measurement for generalised description of track |
| ... | NR | ..... |
| ... | NR | ..... |
| LATITUDE_n | NR | nth trackline latitude of measurement for generalised description of track |
| LONGITUDE_n | NR | nth trackline longitude of measurement for generalised description of track |

Entity Relationship Diagram for BEDMAP Database

Trackline Sortie Data Table

| Sortie-id | Mission-id | Date | DataVol | Latitude_1 | Longitude_1 | Latitude_2 | ... |
| --- | --- | --- | --- | --- | --- | --- | --- |

		Mission Data Table

| Record-id | Sortie-id | Mission-id | Latitude | Longitude | Ice_thickness | Surface_Elev |
| --- | --- | --- | --- | --- | --- | --- |



Mission Summary Table

| Mission-id | Region | Date | Datatype | Platform | Thicmeth | Thicproc | ... |
| --- | --- | --- | --- | --- | --- | --- | --- |

Note:  infers one-to-many relationship, all other relationships are one-to-one